IMPLEMENTATION GUIDE

Multi-Site Network Deployment Automation Framework

Step-by-step instructions for engineers to generate and deploy standardized network configurations

| Document Version | 2.0 |
| --- | --- |
| Date | March 2026 |
| Audience | Network Engineers, Automation Engineers, Team Leads |
| Prerequisites | Python 3.8+, pip, Ansible 2.12+ (optional for automated deployment) |

# Quick Reference – The Entire Workflow in 4 Commands

For experienced engineers who just need the commands. Detailed explanations follow in the step-by-step guide below.

# 1. Install dependencies

pip install openpyxl jinja2

# 2. Validate the Excel workbook

python scripts/validate_inputs.py -i Multi_Site_Deployment_Framework.xlsx

# 3. Generate all site configs

python scripts/generate_configs.py -i Multi_Site_Deployment_Framework.xlsx -o ./output -t ./templates

# 4. Deploy (Ansible – optional, can also deploy manually)

ansible-playbook deploy_site.yml --extra-vars "site=Versah" --check    # dry-run first

ansible-playbook deploy_site.yml --extra-vars "site=Versah"             # actual deploy

## Data Flow: How Excel Becomes Config

| Excel Sheet | → | Feeds Into | Config Section |
| --- | --- | --- | --- |
| Site Inventory | → | hostname, mgmt_ip, ha_role, stack_priority | Hostname, stack config, management SVI |
| IP Addressing | → | svi.ip, svi.mask, svi.vlan_id, gateway | All SVI interfaces with IP addresses |
| VLAN Mapping | → | vlan.id, vlan.name, trunk_vlans | VLAN database, trunk allowed lists |
| Routing | → | route.destination, route.next_hop | Static route statements |
| Security Policies | → | acl rules, zone mappings | Access-lists, firewall rules |

# Deliverable Package Contents

The automation package contains the following files. All paths are relative to the package root.

| File | Type | Purpose |
| --- | --- | --- |
| Multi_Site_Deployment_Framework.xlsx | Excel | Source of truth – 9 sheets with validation, formulas, dashboard |
| scripts/validate_inputs.py | Python | Pre-flight validation: duplicate IPs, CIDR overlap, naming, missing fields |
| scripts/generate_configs.py | Python | Reads Excel → builds variable dictionaries → renders Jinja2 templates |
| templates/switch_config.j2 | Jinja2 | Cisco IOS switch template: VLANs, SVIs, trunks, AAA, SNMP, NTP, STP |
| templates/velocloud_edge.j2 | Jinja2 | VeloCloud Edge template: VCO API JSON: LAN/WAN, HA, business policies, firewall |
| deploy_site.yml | Ansible | Deployment playbook: backup → push → validate → save |
| output/{site}/{hostname}.cfg | Generated | Per-device configs – created by generate_configs.py |

STEP 1   SET UP THE PROJECT

Unzip the automation package and install dependencies

Extract the automation package to your working directory and install the two required Python libraries. No other software is needed for config generation.

# Unzip the deliverable package

unzip Automation_Package.zip

cd package/

# Install Python dependencies (Python 3.8+ required)

pip install openpyxl jinja2

# Optional: Install Ansible for automated deployment (Step 6)

pip install ansible

Verify the directory structure matches:

package/

├── Multi_Site_Deployment_Framework.xlsx

├── scripts/

│   ├── generate_configs.py

│   └── validate_inputs.py

├── templates/

│   ├── switch_config.j2

│   └── velocloud_edge.j2

├── deploy_site.yml

└── output/              (created after generation)

STEP 2   FILL THE EXCEL WORKBOOK

Populate yellow cells only – formulas and dashboard auto-update

Open Multi_Site_Deployment_Framework.xlsx and fill the sheets in the order listed below. Only edit yellow-highlighted cells. Green cells contain formulas and will auto-calculate.

### Sheet 1: Site Inventory (one row per device)

This is the master device registry. Add one row for every physical device at the site.

| Column | Field | Example Value | Notes |
| --- | --- | --- | --- |
| B | Site Name | Versah | Must match across all sheets |
| C | Site Location | CHE – Remote | Physical location |
| D | Device Hostname | sw-ver-us-mdf-1 | Follow naming convention (see Naming Standards tab) |
| E | Device Type | Switch | Dropdown: Switch / SD-WAN / Firewall / AP |
| F | Management IP | 10.246.233.225 | Must be unique across all devices |
| M | HA Role | Stack Master | Dropdown: Primary / Secondary / Stack Master |
| N | Deployment Status | Planning | Dropdown: Planning → Production |

### Sheet 2: IP Addressing (one row per subnet)

Define every subnet for the site. The Gateway IP becomes the SVI IP address in the generated switch config.

| Column | Field | Example Value | Notes |
| --- | --- | --- | --- |
| C | Site CIDR Block | 10.246.232.0/23 | The site supernet |
| D | VLAN ID | 10 | Must match VLAN Mapping sheet |
| F | Subnet (CIDR) | 10.246.232.0/24 | Validated for overlap |
| G | Gateway IP | 10.246.232.1 | Becomes the SVI ip address in config |
| K | Subnet Mask | 255.255.255.0 | Dotted decimal for SVI config |

### Sheet 3: VLAN Mapping (standard template)

Pre-filled with the 8 standard VLANs (3, 5, 9, 10, 18, 188, 192, 193). Adjust VLAN names or IDs only if your site deviates from the standard template. For most sites, no changes are needed here.

### Sheet 4: Routing (one row per route per device)

The most critical entry is the default route for the switch pointing to the SD-WAN Transfer VLAN gateway:

Device:      sw-ver-us-mdf-1

Destination: 0.0.0.0/0

Next Hop:    10.246.233.162   # SD-WAN IP on Transfer VLAN

Protocol:    Static

Interface:   Vlan3

STEP 3   VALIDATE INPUTS

Run the validator to catch errors before generating configs

The validation script checks for: duplicate IPs, duplicate hostnames, CIDR overlaps between subnets at the same site, naming convention compliance, missing required fields, and VLAN ID range violations (1–4094).

python scripts/validate_inputs.py -i Multi_Site_Deployment_Framework.xlsx

Expected output on a clean workbook:

============================================================

MULTI-SITE DEPLOYMENT FRAMEWORK - INPUT VALIDATOR

File: Multi_Site_Deployment_Framework.xlsx

============================================================

[1/4] Validating Site Inventory...

[2/4] Validating IP Addressing Plan...

[3/4] Validating VLAN Mapping...

[4/4] Validating Routing Table...

============================================================

RESULT: PASSED - All validations clean

============================================================

If errors are found, fix them in the Excel workbook and re-run. ERRORS are blocking (must fix before proceeding). WARNINGS are informational (review but not blocking).

STEP 4   GENERATE CONFIGURATIONS

Excel → Python → Jinja2 → per-device .cfg files

The generator reads all Excel sheets, builds a variable dictionary per device, and renders the appropriate Jinja2 template. Output is organized into folders by site name.

# Generate ALL sites

python scripts/generate_configs.py \

-i Multi_Site_Deployment_Framework.xlsx \

-o ./output \

-t ./templates

You can also generate for a single site or single device:

# Generate ONE site only

python scripts/generate_configs.py -i Framework.xlsx --site "Versah"

# Generate ONE device only

python scripts/generate_configs.py -i Framework.xlsx --device "sw-ver-us-mdf-1"

Expected output:

============================================================

MULTI-SITE CONFIG GENERATOR

Input:  Multi_Site_Deployment_Framework.xlsx

Output: ./output

============================================================

[INFO] Loaded 10 devices from Site Inventory

[OK] sw-ver-us-mdf-1.cfg (Switch)  → ./output/Versah/

[OK] sd-ver-us-mdf-1.json (SD-WAN)  → ./output/Versah/

[OK] sd-ver-us-mdf-2.json (SD-WAN)  → ./output/Versah/

[OK] sw-nob-ch-mdf-1.cfg (Switch)  → ./output/Nobel_Biocare/

[OK] sd-nob-ch-mdf-1.json (SD-WAN)  → ./output/Nobel_Biocare/

[OK] sd-nob-ch-mdf-2.json (SD-WAN)  → ./output/Nobel_Biocare/

SUMMARY: 6 configs generated, 4 skipped

============================================================

The output directory structure will look like:

output/

├── Versah/

│   ├── sw-ver-us-mdf-1.cfg   (283 lines – full switch config)

│   ├── sd-ver-us-mdf-1.json   (VeloCloud Edge – VCO API spec)

│   └── sd-ver-us-mdf-2.json   (VeloCloud Edge – VCO API spec)

└── Nobel_Biocare/

├── sw-nob-ch-mdf-1.cfg

├── sd-nob-ch-mdf-1.json

└── sd-nob-ch-mdf-2.json

STEP 5   REVIEW & REPLACE PLACEHOLDERS

Inspect output, replace secrets, peer review

Open the generated .cfg files and verify the output. Some values are intentionally left as placeholders (angle-bracket tokens) because they contain secrets that should not be stored in the Excel workbook.

# Preview a generated config

cat output/Versah/sw-ver-us-mdf-1.cfg | head -40

# Find all remaining placeholders across all generated configs

grep -rn "<.*>" output/

Placeholders that require replacement:

| Placeholder | Replace With | Where to Get It |
| --- | --- | --- |
| <ENABLE_SECRET> | Switch enable password | Password vault / team lead |
| <ADMIN_PASSWORD> | Local admin password | Password vault / team lead |
| <TACACS_SERVER_IP> | ISE/TACACS server IP (e.g. 10.x.x.x) | ISE admin / Network team |
| <TACACS_KEY> | TACACS shared secret | ISE admin |
| <RO_COMMUNITY> | SNMP read-only community string | Monitoring team / NOC |
| <RW_COMMUNITY> | SNMP read-write community string | Monitoring team / NOC |
| <SOLARWINDS_IP> | Solarwinds polling server IP | NOC team |
| <SYSLOG_SERVER> | Central syslog server IP | Systems team |
| <NTP_SERVER_1>, <NTP_SERVER_2> | NTP server IPs | Systems team / AD servers |

Pro tip: For production use, create a global_vars.json file with these secrets and modify generate_configs.py to load it. This keeps secrets out of the Excel workbook entirely and enables different values per environment.

STEP 6   DEPLOY TO DEVICES

Push configs via Ansible or manually via SSH

### Option A: Automated Deployment (Ansible)

The Ansible playbook handles backup, config push, and post-deployment validation in a single run. Always run in check mode first.

# DRY-RUN first (check mode – no changes made to devices)

ansible-playbook deploy_site.yml \

-i inventory.yml \

--extra-vars "site=Versah" \

--check

# ACTUAL deployment (pushes configs and saves)

ansible-playbook deploy_site.yml \

-i inventory.yml \

--extra-vars "site=Versah"

# Deploy SINGLE device only

ansible-playbook deploy_site.yml \

-i inventory.yml \

--limit sw-ver-us-mdf-1

The playbook automatically performs: backup current running-config → push new config → validate VLANs, interfaces, routing, STP → save startup-config → write validation report.

### Option B: Manual Deployment (SSH)

If Ansible is not available, copy the config file to the device and apply manually:

# SCP the config file to the device

scp output/Versah/sw-ver-us-mdf-1.cfg admin@10.246.233.225:

# SSH to the device

ssh admin@10.246.233.225

# Apply the config (on the device CLI)

copy flash:sw-ver-us-mdf-1.cfg running-config

write memory

STEP 7   POST-DEPLOYMENT VALIDATION

Verify everything is working, update Excel status

Run these verification commands on each deployed device to confirm the configuration was applied correctly:

| Command | What to Verify |
| --- | --- |
| show switch | Stack members present, correct priorities, SSO mode active |
| show vlan brief | All 8 VLANs created with correct names (VER-USER, VER-WIFI, etc.) |
| show ip interface brief | SVIs up/up with correct IP addresses from IP Addressing sheet |
| show interfaces trunk | Uplink ports (gi-1/0/1-2, gi-2/0/1-2) trunking all VLANs |
| show spanning-tree summary | Switch is root bridge (priority 4096) for all VLANs |
| show ip route | Default route 0.0.0.0/0 via SD-WAN Transfer VLAN gateway |
| show run ¦ section tacacs | TACACS server configured, AAA authentication active |
| show snmp | SNMP community configured, traps enabled, Solarwinds host set |
| show ntp status | NTP synchronized with configured servers |

After successful validation, update the Excel workbook:

- In the Site Inventory sheet, change column N (Deployment Status) from "Configured" to "Validated"

- After production burn-in period, change status to "Production"

- The Dashboard sheet auto-updates to reflect the new status

- Commit the generated .cfg files to Git for version control and audit trail

# Adding a New Site

To deploy a new site, repeat the same 7-step process with new data. The templates and automation scripts remain unchanged.

- Copy existing rows from a working site (e.g. Versah) in each Excel sheet

- Change Site Name, Location, hostnames, and IP addresses to match the new site

- VLAN Mapping typically stays the same (standard template across all sites)

- Run validate → generate → review → deploy as before

- Total time: approximately 4 hours vs 25 hours manual (84% reduction)

# Troubleshooting

| Problem | Cause | Solution |
| --- | --- | --- |
| ModuleNotFoundError: openpyxl | Python dependency not installed | Run: pip install openpyxl jinja2 |
| Validator shows [ERROR] Duplicate IP | Two devices share the same management IP in Site Inventory | Check column F for duplicate values, fix in Excel |
| Validator shows [ERROR] Subnet overlap | Two subnets in IP Addressing sheet overlap for the same site | Review CIDR blocks in column F, ensure no overlap |
| Generator shows [SKIP] for a device | No Jinja2 template exists for that device type (e.g. Access Point) | Expected for APs – they use WLC templates. Add .j2 template for new types |
| Generated config has wrong VLAN IDs | VLAN Mapping sheet has wrong site filter or missing VLANs | Ensure VLAN Mapping column B says "ALL SITES" or the exact site name |
| SVI IP missing in generated config | IP Addressing sheet missing row for that VLAN at that site | Add a row with VLAN ID, Gateway IP, and Subnet Mask |

Support

For questions about the automation framework, config templates, or deployment procedures, contact the Network Infrastructure team. For issues with the Excel validation or Python scripts, refer to the inline code comments or raise a ticket in ServiceNow.